zV

PRODUCT SPECFICATION   产品规格书

| Project Code/项目代码 | CMB04040173 |
| --- | --- |
| Version NO./ 版本号 | A0 |
| Product Name/ 产品名称 | / |
| Model/ 型号 | 14.8V20AH |

| Drafted By/ Date 拟定/日期 | 20250716 |
| --- | --- |
| Checked By/ Date 审核/日期 | 20250716 |
| Approval By/ Date审批/日期 | 20250716 |

| Customer Approval/ 客户确认 | Checked By/Date by审核/日期 | Approval By/ Date 审批/日期 |
| --- | --- | --- |
| Company Seal/公章 |  |  |

Tel : +86 158 1732 3917

Email :cherry@cmbatteries.com

CMB Headquarters：
Rm.1216 ,

Baoshan Times Bldg,

Minqiang Community,

Minzhi St.,Longhua Dist.,
Shenzhen

GuangdongChina

CMB Facotry：

Rm.702,7/F,4 BLDG,NO.160,

Tianxin Luyuan Rd.,

Keyuan City,Tangxia Town,

Dongguan,

Guangdong,China

Date 日期：

20250716

Project Name项目名：

CMB04040173

Manufacturer

生产制造商

CM Batteries Co.,Ltd

Customer ID

客户编码：

ABXXX

page：1

| Amendment Record /修订记录 |  |  |  |  |
| --- | --- | --- | --- | --- |
| NO. 序号 | Version 版本 | Description 描述 | Signature 签名 | Date 日期 |
| 1 | A0 | 初版拟定 | WHH | 20250716 |

Tel : +86 158 1732 3917

Email :cherry@cmbatteries.com

CMB Headquarters：
Rm.1216 ,

Baoshan Times Bldg,

Minqiang Community,

Minzhi St.,Longhua Dist.,
Shenzhen

GuangdongChina

CMB Facotry：

Rm.702,7/F,4 BLDG,NO.160,

Tianxin Luyuan Rd.,

Keyuan City,Tangxia Town,

Dongguan,

Guangdong,China

Date 日期：

20250716

Project Name项目名：

CMB04040173

Manufacturer

生产制造商

CM Batteries Co.,Ltd

Customer ID

客户编码：

ABXXX

page：2

PRODUCT SPECFICATION  产品规格书

Requirements for Measuring Instruments and Equipment

TABLE OF CONTENTS/目录

- Amendment Records版本变更记录 ....................................................................................................2

2. TABLE OF CONTENTS目录 .....................................................................................................................3

2.1. Standard标准 ........................................................................................................................................4

2.2. Production Description 产品描述 ..................................................................................................4

3. Scope范围 ..............................................................................................................................................5

4. Product parameters产品参数 ..............................................................................................................5

5. Material Sheet材料表 ............................................................................................................................6

6. Battery Pack Reliability Test电池组可靠性测试 ...........................................................................6

6.1. Testing Condition测试条件 .................................................................................................................................6

6.2. Requirements for Measuring Instruments and Equipment

测量仪器设备要求 ...........................................................................................................................................................7

6.3. Specifications for testing electrical performance parameters

电气性能参数测试规范 ....................................................................................................................................................8

6.3.1.Charge Test充电测试 ..........................................................................................................8

6.3.2. Discharge Test放电试验 ....................................................................................................8

6.3.3. Rated Capacity额定容量 ..................................................................................................9

6.3.4.Charge Retention电荷保持能力 .....................................................................................9

6.3.5. Internal Impedance内阻 ....................................................................................................9

6.3.6. Life Cycle Test 循环寿命测试 ............................................................................................10

6.4. Environmental Performance环境性能 ........................................................................................10

6.4.1. High Temperature Performance高温性能 .................................................................10

6.4.2. Low Temperature Performance低温性能 ..................................................................11

6.4.3. Constant Temperature and Humidity Performance恒温恒湿性能 ................11

6.4.4. Heat Test热冲击 ....................................................................................................................12

6.4.5.Vibration Test振动测试 ........................................................................................................12

6.5. Battery Safety Protection Performance Test Range

电池安全保护性能测试范围 ..........................................................................................................................................12

6.5.1.Overcharge Protection过度充电保护 .............................................................................12

6.5.2. Over discharge Test过度放电测试 ..................................................................................13

6.5.3. External Short-circuit Test外部短路测试 ......................................................................13

7. Transportation, Storage运输、存储 .....................................................................................................13

7.1. Transportation运输 .........................................................................................................................13

7.2. Storage存储 .......................................................................................................................................14

8. Handling Instructions操作说明 ...............................................................................................................14

9. Amendment of this Specification产品规格书的修订..........................................................15

8. Transportation,Storage,Mark and Package运输,存储,标志和包装示意图...........................18

- 9. Handling Instruction操作指南....................................................................................................19

9. Amendment of this Specification本规范的修订..........................................................................20

PRODUCT SPECFICATION   产品规格书

Tel : +86 158 1732 3917

Email :cherry@cmbatteries.com

CMB Headquarters：
Rm.1216 ,

Baoshan Times Bldg,

Minqiang Community,

Minzhi St.,Longhua Dist.,
Shenzhen

GuangdongChina

CMB Facotry：

Rm.702,7/F,4 BLDG,NO.160,

Tianxin Luyuan Rd.,

Keyuan City,Tangxia Town,

Dongguan,

Guangdong,China

Date 日期：

20250716

Project Name项目名：

CMB04040173

Manufacturer

生产制造商

CM Batteries Co.,Ltd

Customer ID

客户编码：

ABXXX

page：3

Product Summary 产品概要

1. Standard 依据标准

This product complies with National Standard GB 31241-2022《Lithium-ion Battery General Specifications》of PRC.

# 本产品依据标准：中华人民共和国国家标准 GB 31241-2022《便携式电子产品用锂离子电池和电池组安全要求》。

2. Production description 产品描述

a) Cell 电芯：INR21700 5000mAh

b) Structure Drawing 电池结构图：

PRODUCT SPECFICATION   产品规格书

Tel : +86 158 1732 3917

Email :cherry@cmbatteries.com

CMB Headquarters：
Rm.1216 ,

Baoshan Times Bldg,

Minqiang Community,

Minzhi St.,Longhua Dist.,
Shenzhen

GuangdongChina

CMB Facotry：

Rm.702,7/F,4 BLDG,NO.160,

Tianxin Luyuan Rd.,

Keyuan City,Tangxia Town,

Dongguan,

Guangdong,China

Date 日期：

20250716

Project Name项目名：

CMB04040173

Manufacturer

生产制造商

CM Batteries Co.,Ltd

Customer ID

客户编码：

ABXXX

page：4

- 3. Scope范围

- This specification applies to the technical requirements, test items, testing rules, safety warnings, packaging, transportation and storage of lithium-ion battery packs produced by CM BATTERIES.

- 本规格书适用于CM BATTERIES CO.,Ltd生产的锂离子电池组的技术要求、测试项目、测试规则。4. Product parameters产品参数

| 0NO. 序号 | Item项目 | Parameter 参数 | Remark 备注 |
| --- | --- | --- | --- |
| 1 | Nominal Voltage标称电压 | 14.8V | 3.7V/Cell |
| 2 | Nominal Capacity标称容量 | 20Ah | @0.2C |
| 3 | Minimum Capacity最小容量 | 19.4Ah |  |
| 4 | Shipment Voltage装运电压 | / |  |
| 5 | Initial Internal Impedance初始内阻 | ≤130mΩ | AC 1KHZ |
| 6 | Standard Charge Voltage标准充电电压 | 16.8V | 4.2V/Cell |
| 7 | Standard Charge Current标准充电电流 | 4A | @0.2C |
| 8 | Max. Charge Current最大充电电流 | 7A |  |
| 9 | StandardDischargeCurrent标准放电电流 | 4A | @0.2C |
| 10 | Max. Discharge Current最大放电电流 | 10A |  |
| 11 | Burst Discharge Current突发放电电流 | 30-40A | ≤20ms |
| 12 | Discharge Cut-off Voltage放电截止电压 | 10.8V | 2.7V/Cell |
| 13 | Cycle life循环寿命 | ≥400cycles | Retention: ≥70% |
| 14 | Charge Temperature充电温度 | 0°C~+45°C | Standard Charge |
| 15 | Discharge Temperature放电温度 | -20°C ~+60°C | Standard Discharge |
| 16 | Storage Temperature储存温度 | -20°C~ +35°C | ≤3 months |
|  |  | -20°C~ +20°C | ≤1 year |
| 17 | Weight重量 | About 1.2 | kg |
| 18 | Special Request 特殊技术要求 |  | ​​​ |

Tel : +86 158 1732 3917

Email :cherry@cmbatteries.com

CMB Headquarters：
Rm.1216 ,

Baoshan Times Bldg,

Minqiang Community,

Minzhi St.,Longhua Dist.,
Shenzhen

GuangdongChina

CMB Facotry：

Rm.702,7/F,4 BLDG,NO.160,

Tianxin Luyuan Rd.,

Keyuan City,Tangxia Town,

Dongguan,

Guangdong,China

Date 日期：

20250716

Project Name项目名：

CMB04040173

Manufacturer

生产制造商

CM Batteries Co.,Ltd

Customer ID

客户编码：

ABXXX

page：5

PRODUCT SPECFICATION   产品规格书

Tel : +86 158 1732 3917

Email :cherry@cmbatteries.com

CMB Headquarters：
Rm.1216 ,

Baoshan Times Bldg,

Minqiang Community,

Minzhi St.,Longhua Dist.,
Shenzhen

GuangdongChina

CMB Facotry：

Rm.702,7/F,4 BLDG,NO.160,

Tianxin Luyuan Rd.,

Keyuan City,Tangxia Town,

Dongguan,

Guangdong,China

| NO. 序号 | Item 项目 | Description 描述 | QTY 数量 | Unit 单元 | Remark 备注 |
| --- | --- | --- | --- | --- | --- |
| 1 | Battery cell电芯 | INR21700 5000mAh | 16 | PCS |  |
| 2 | BMS保护板 | / | 1 | PCS |  |

​​

6. Battery Pack Reliability Test电池组可靠性测试

6.1.Testing Condition测试条件

Each test should be carried out under the standard atmospheric pressure

conditions of the test unless otherwise specified.

除非另有规定，否则每次试验应在试验的标准大气压力条件下进行。

5. Material Sheet材料表

PRODUCT SPECFICATION   产品规格书

Date 日期：

20250716

Project Name项目名：

CMB04040173

Manufacturer

生产制造商

CM Batteries Co.,Ltd

Customer ID

客户编码：

ABXXX

page：6

Temperature温度20°C~30°C
Relative Humidity相对湿度45%~75%
Atmospheric Pressure大气压力86KP~106KP

Pre-cycle预循环:

Do a pre-cycle before the test, charge at 0.2C5A at an ambient temperature of25±5°C, and when the battery terminal reaches the charge limit voltage, put it on hold for 0.5 to 1 hour, and then discharge at  0.2C5A to the cut-off voltage.

测试前进行预循环，在25±5°C的环境温度下以0.2C5A充电，当电池端子达到充电极限电压时，将其保持0.5至1小时，然后以0.2C5A放电至截止电压。

6.2. Requirements for measuring instruments and equipment测量仪器设备要求

•  The accuracy of the instrument for measuring voltage should not be lower

than 0.5, and the internal resistance should not be less than 10KΩ/V.

测量电压的仪器的精度不应低于大于0.5内阻不应小于10KΩ/V

•  The accuracy of the instrument for measuring current shall not be lower than

0.5 grade.

电流测量仪器的精度不应低于0.5级

•  The accuracy of the instrument for measuring time should not be less than±0.1%.

仪器测量时间的精度不应低于±0.1%

•  The accuracy of the instrument for measuring current shall not be lower than

0.5 grade .

电流测量仪器的精度不应低于0.5级

•  The current of the constant current source is constant and adjustable, and itscurrent variation should be within ±1% during charging or discharging.

恒流源的电流是恒定和可调的，在充电或放电过程中其电流变化应在±1%以内

•  The voltage of the constant voltage source is adjustable, and its voltage

variation range is±0.5%.

恒压源的电压是可调的，其电压变化范围为±0.5%

•  The accuracy of the dimension measuring instrument shall not be less than

0.02mm.

尺寸测量仪器的精度不应低于0.02毫米

•  The sensitivity of the weighing instrument should not be less than 0.1g.

称重仪器的灵敏度不应低于0.1g

6.3. Specifications for testing electrical performance parameters电气性能参数测试规范

PRODUCT SPECFICATION   产品规格书

Tel : +86 158 1732 3917

Email :cherry@cmbatteries.com

CMB Headquarters：
Rm.1216 ,

Baoshan Times Bldg,

Minqiang Community,

Minzhi St.,Longhua Dist.,
Shenzhen

GuangdongChina

CMB Facotry：

Rm.702,7/F,4 BLDG,NO.160,

Tianxin Luyuan Rd.,

Keyuan City,Tangxia Town,

Dongguan,

Guangdong,China

Date 日期：

20250716

Project Name项目名：

CMB04040173

Manufacturer

生产制造商

CM Batteries Co.,Ltd

Customer ID

客户编码：

ABXXX

page：7

6.3.1.Charge Test充电测试
•  Test Method测试方法:

•  Standard charging标准充电:
  charge at 0.2C5A constant current at 25±5°C, change to constant voltage

charging when the power terminal voltage reaches the limit voltage, stop

charging until the charging current is less than or equal to 0.02C5A, and the

time does not exceed 8 hours.

在25±5℃下以0.2C5A恒定电流充电，变为恒定电压当电源端电压达到极限电压时充电，停止充电直到充电电流小于或等于0.02C5A，并且时间不超过8小时。

•  Fast charging快速充电:

Charge with a constant current of7 A at 25±5°C, and change to constant
voltage charging when the voltage at the power supply terminal reaches the
limit voltage, until the charging current is less than or equal to 0.02C5A, and

the time does not exceed 8 hours, then stop charging.

在25±5°C下用7 A的恒定电流充电，并变为恒定当电源端子处的电压达到限制电压，直到充电电流小于或等于0.02C5A，并且时间不超过8小时，然后停止充电。

•  Approval Standard合格标准

Any charging mode can be used, and the charging current can reach a state of less than 0.02C5A within 8 hours.

可以使用任何充电模式，充电电流可以达到8小时内低于0.02C5A。

6.3.2. Discharge Test放电试验

•  Test Method测试方法:

•  Standard discharge标准放电:

Discharge at a constant current of 0.2C5A to the discharge cut-off voltage

under the condition of 25±5°C; the discharge time is not less than 5 hours;

以0.2C5A的恒定电流放电至放电截止电压在25±5℃的条件下；放电时间不少于5小时

•  Rapid discharge快速放电:

Discharge at a constant current of 10A to the discharge cut-off voltage

under the condition of 25±5°C; the discharge time is not less than 105 minutes.

在25±5℃的条件下，以10A的恒定电流放电至放电截止电压；放电时间不小于 105 分钟。

•  Discharge temperature放电温度:

The discharge process can be carried out at -20~+60°C  , without other bad

conditions.

放电过程可以在-20~+60℃下进行，没有其他不良条件。

Tel : +86 158 1732 3917

Email :cherry@cmbatteries.com

CMB Headquarters：
Rm.1216 ,

Baoshan Times Bldg,

Minqiang Community,

Minzhi St.,Longhua Dist.,
Shenzhen

GuangdongChina

CMB Facotry：

Rm.702,7/F,4 BLDG,NO.160,

Tianxin Luyuan Rd.,

Keyuan City,Tangxia Town,

Dongguan,

Guangdong,China

PRODUCT SPECFICATION   产品规格书

Date 日期：

20250716

Project Name项目名：

CMB04040173

Manufacturer

生产制造商

CM Batteries Co.,Ltd

Customer ID

客户编码：

ABXXX

page：8

6.3.3. Rated Capacity额定容量
•  Test Method测试方法:
  At an ambient temperature of 25±5°C, charge the battery according to the

method described in  Standard charging;

在25±5℃的环境温度下，按照“标准充电”中所述的方法对电池进行充电；

After charging, place it at the same temperature of 25±5°C for 1 to 2 hours;

充电后，将其置于25±5℃的相同温度下1至2小时；

Under the condition of ambient temperature of 25±5°C, discharge the battery
according to the method described in Standard discharge.

在环境温度为25±5℃的条件下，按照“标准放电”中所述的方法对电池进行放电；

•  Approval Standard合格标准:

The standard discharge time is greater than 5  hours, and the rapid discharge

time is greater than  105   minutes.

标准放电时间大于5小时，快速放电时间大于 105 分钟。

6.3.4.Charge Retention电荷保持能力

•  Test Method测试方法:

At an ambient temperature of 25±5°C, charge the battery according to the

method described in Standard charging;

在25±5℃的环境温度下，按照“标准充电”中所述的方法对电池进行充电；

Under the same ambient temperature of 25±5°C, the battery should be stored

for 30 days in an open circuit state;

在25±5℃的相同环境温度下，电池应在开路状态下储存30天；

Under Under the same ambient temperature of 25±5°C, perform a standard
discharge operation on the battery.

在环境温度为25±5℃的条件下，按照“标准放电”中所述的方法对电池进行放电。

•  Approval Standard合格标准:

Standard discharge time>4.0 hours; no other failure conditions during discharge

标准放电时间>4.0小时；期间无其他故障情况。

6.3.5. Internal Impedance内阻

•  Test Method测试方法:

At an ambient temperature of 25±5°C, use a dedicated battery comprehensive

tester to measure the internal resistance between the positive and negative

electrodes at the battery interface. The measurement should not be less than

5 times, and the corresponding valid results should be averaged.

在25±5°C的环境温度下，使用专用的电池内阻测试仪测量电池的正极和负极之间的内阻。测量次数

不应小于5次，对有效测量结果计算平均值。

•  Approval Standard合格标准:

Internal Impedance(内部阻抗) <130mΩ

PRODUCT SPECFICATION   产品规格书

Tel : +86 158 1732 3917

Email :cherry@cmbatteries.com

CMB Headquarters：
Rm.1216 ,

Baoshan Times Bldg,

Minqiang Community,

Minzhi St.,Longhua Dist.,
Shenzhen

GuangdongChina

CMB Facotry：

Rm.702,7/F,4 BLDG,NO.160,

Tianxin Luyuan Rd.,

Keyuan City,Tangxia Town,

Dongguan,

Guangdong,China

Date 日期：

20250716

Project Name项目名：

CMB04040173

Manufacturer

生产制造商

CM Batteries Co.,Ltd

Customer ID

客户编码：

ABXXX

page：9

6.3.6. Life Cycle Test 循环寿命测试
•  Test Method测试方法:
  The whole process of this test should be carried out at an ambient

temperature of 25±5°C;

本试验的整个过程应在25±5°C的环境温度下进行

The cycle process includes two actions of charging and discharging, the cycle

starts as charging, and the cycle ends as discharging;

循环过程包括充电和放电两个动作，循环以充电开始，以放电结束；

Charge with 0.2C5A constant current, change to constant voltage charging

when the battery terminal voltage reaches the charging limit voltage, and stop

charging when the charging current is less than or equal to 0.02C5A;

以0.2C5A电流恒流充电，但达到充电限制电压时，改为恒压充电；当充电电流小于或等于0.02C5A

时停止充电；

After leaving it aside for 0.5 hours to 1 hour, discharge it with a constant current

of 0.2 C5A. When the terminal voltage of the battery reaches the cut-off

voltage,the discharge ends;

将其放置0.5至1小时后，以0.2 C5A电流恒流放电。当电池的电压达到截止电压时，放电结束；

After resting for 0.5 hours to 1 hour, enter the next charge and discharge cycle

action.

放置0.5小时至1小时后，进入下一次充电和放电循环行动。

•  Approval Standard合格标准:

Cycle Life 400 times, remaining capacity ≥ 70% of initial capacity

循环寿命400次，剩余容量≥初始容量的70%。

6.4. Environmental Performance环境性能

6.4.1. High Temperature Performance高温性能

•  Test Method测试方法:

After the standard 0.2C5A charge is completed at an ambient temperature of
25±5°C, put the battery in a high-temperature box at (+60±2)°C and keep the
temperature constant for 4 hours; discharge at a constant current of 0.2A to
the discharge cut-off voltage, and the discharge ends.

在环境温度为25±5°C的情况下，以0.2C5A电流标准充电结束后，将电池放在(+60±2)°C的高温箱中，并保持温度不变4小时。再以0.2C5A的恒定电流放电至放电截止电压，放电结束。

After the end of the test, place it for 2 hours under the condition of 25±5°C, then make an appearance inspection.

试验结束后，在25±5℃的情况下放置2小时，然后进行外观检查。

Under an ambient temperature of 25±5°C, perform a standard
discharge operation on the battery.

在环境温度为25±5°C的情况下，对电池进行标准放电操作。

PRODUCT SPECFICATION  产品规格书

Tel : +86 158 1732 3917

Email :cherry@cmbatteries.com

CMB Headquarters：
Rm.1216 ,

Baoshan Times Bldg,

Minqiang Community,

Minzhi St.,Longhua Dist.,
Shenzhen

GuangdongChina

CMB Facotry：

Rm.702,7/F,4 BLDG,NO.160,

Tianxin Luyuan Rd.,

Keyuan City,Tangxia Town,

Dongguan,

Guangdong,China

Date 日期：

20250716

Project Name项目名：

CMB04040173

Manufacturer

生产制造商

CM Batteries Co.,Ltd

Customer ID

客户编码：

ABXXX

page：10

•  Approval Standard合格标准:

the discharge capacity is not less than the minimum capacity of the product;

the battery has no deformation, leakage, smoke or fire.

电池的放电容量不低于产品的最小容量；电池没有变形、漏液、冒烟或起火。

6.4.2. Low Temperature Performance低温性能
•  Test Method测试方法:
  After the standard 0.2C5A charging is completed at an ambient temperature 
  of 25±5°C, put the battery in a low-temperature box at (-20±2)°C for 16 to 24
  hours dischC5arge it at a constant current of 0.2C5A to 10.8 V, and the 
  discharge ends.

在环境温度为25±5°C的情况下完成标准的0.2C5A充电后，将电池放在(-20±2)°C的低温箱中16

至24小时，以0.2C5A的恒定电流放电至10.8 V，然后放电结束。

After the end of the test, place it for 2 hours under the condition of 25±5°C,

then make an appearance inspection.

试验结束后，在25±5℃的条件下放置2小时，然后进行外观检查。

•  Approval Standard合格标准:

The battery is at -20±2 °C, and the discharge capacity is not less than 50%*

nominal capacity; the battery has no deformation, leakage, smoke or fire.

电池处于-20±2 ℃，放电容量不低于50% *标称容量；电池没有变形、泄漏、冒烟或起火。

6.4.3. Constant Temperature and Humidity Performance恒温恒湿性能

•  Test Method测试方法:

After the standard 0.2C5A charging at an ambient temperature of 25±5°C, put

the battery in a constant temperature and humidity box at 40°C and 95%RH for    48 hours, and then restore it for 2 hours at an ambient temperature of 25±5°C;

use a constant current of 0.2C5A Discharge to10.8 V, the discharge ends;

在环境温度为25±5°C的情况下进行标准的0.2C5A充电后，将电池放在40°C和95%RH的恒温恒湿箱中48小时，然后在环境温度为25±5°C的情况下搁置2小时；再使用0.2C5A的恒定电流放电至10.8 V，放电结束；

After the end of the test, place it for 2 hours under the condition of 25±5°C, then make an appearance inspection.

试验结束后，在25±5℃的条件下放置2小时，然后进行外观检查。

•  Approval Standard合格标准:

There should be no obvious deformation, rust, smoke or explosion on the

appearance of the battery. Discharge at 0.2C5A, discharge capacity 60%*

nominal capacity.

电池的外观不应有明显的变形、生锈、冒烟或爆炸等现象。电池的外观不应有明显的变形、生锈

冒烟或爆炸。在0.2C5A时放电，放电容量为60%*标称容量。

PRODUCT SPECFICATION   产品规格书

Tel : +86 158 1732 3917

Email :cherry@cmbatteries.com

CMB Headquarters：
Rm.1216 ,

Baoshan Times Bldg,

Minqiang Community,

Minzhi St.,Longhua Dist.,
Shenzhen

GuangdongChina

CMB Facotry：

Rm.702,7/F,4 BLDG,NO.160,

Tianxin Luyuan Rd.,

Keyuan City,Tangxia Town,

Dongguan,

Guangdong,China

Date 日期：

20250716

Project Name项目名：

CMB04040173

Manufacturer

生产制造商

CM Batteries Co.,Ltd

Customer ID

客户编码：

ABXXX

page：11

6.4.4. Heat Test热冲击

•  Test Method测试方法:

After standard 0.2C5A charging at ambient temperature of 25±5°C;

在环境温度为25±5°C的情况下进行标准的0.2C5A充电；

Put the sample into the hot box, raise the temperature to 130°C±2°C at a rate

of (5°C±2°C)/min and keep it warm for 30min;

将样品放入恒温箱，以(5℃±2℃)/分钟的速度升温至130℃±2℃，保温30分钟；

•  Approval Standard合格标准:

No fire, no explosion.

不起火，不爆炸。

6.4.5.Vibration Test振动测试
•  Test Method测试方法:
  at an ambient temperature of 25±5°C, 0.2C5A charging at ambient temperature

with GB40165-2021 standard and the parameters of Sine Vibration Test:

在环境温度为25±5°C的情况下，以标准0.2C5A充满电后，按GB40165-2021标准，按下表参数

进行正弦振动测试。

•  Approval Standard合格标准:

No fire, no explosion, no leakage.

不起火，不爆炸，无漏液。

6.5. Battery Safety Protection Performance Test Range电池安全保护性能测试范围

6.5.1.Overcharge Protection过度充电保护

•  Test Method测试方法:

at an ambient temperature of 25±5°C, 0.2C5A charging at ambient temperature

在环境温度25±5°C下，以0.2C5A 标准充电方式充满电；

2C5A current, 2 times the standard voltage charging for 8 hours, and then

proceed as follows:

以2C5A电流，2倍标准电压，充电8小时，然后按以下步骤进行：

Standard discharge to termination voltage;标准放电至终止电压

Standard charge to limit voltage; 标准充电至截止电压

Tel : +86 158 1732 3917

Email :cherry@cmbatteries.com

CMB Headquarters：
Rm.1216 ,

Baoshan Times Bldg,

Minqiang Community,

Minzhi St.,Longhua Dist.,
Shenzhen

GuangdongChina

CMB Facotry：

Rm.702,7/F,4 BLDG,NO.160,

Tianxin Luyuan Rd.,

Keyuan City,Tangxia Town,

Dongguan,

Guangdong,China

Date 日期：

20250716

Project Name项目名：

CMB04040173

Manufacturer

生产制造商

CM Batteries Co.,Ltd

Customer ID

客户编码：

ABXXX

page：12

PRODUCT SPECFICATION   产品规格书

Tel : +86 158 1732 3917

Email :cherry@cmbatteries.com

CMB Headquarters：
Rm.1006 10F

Hengbo Bldg

Longping Community

Dalang St.,Longhua Dist
Shenzhen

GuangdongChina

CMB Facotry：

8 Floor,5 Building

Qinggu Intelligent 
Manufacturing

Park,Tangxia Town
Dongguan

Guangdong,China

Date 日期：

20250716

Project Name项目名：

CMB04040173

Manufacturer

生产制造商

CM Batteries Co.,Ltd

Customer ID

客户编码：

ABXXX

Standard discharge to termination voltage;标准放电至终止电压

•  Approval Standard合格标准:

No fire, no explosion. Battery voltage < overcharge protection voltage.

不起火，不爆炸。电池电压<过充电保护电压。

6.5.2. Over discharge Test过度放电测试

•  Test Method测试方法:

After the standard 0.2C5A charging at ambie

Under the condition of ambient temperature 25±5°C,the battery is discharged

To the cut-off voltage at 0.2C5A and connected to an external (30×4) Ω load

for 24hours. Then proceed as follows:

在环境温度为25±5°C的条件下，在0.2C5A的条件下将电池放电到截止电压，并与外部（30×4)Ω负

载连接24小时。然后进行如下操作：

Standard charge to limit voltage;标准充电至截止电压

Standard discharge to termination voltage;标准放电至终止电压

at an ambient temperature of 25±5°C,, discharge at 0.01 C5A until the discharge

current is 0A

在25±5°C环境温度下，以0.01 C5A放电，直到放电电流为0A

The battery should not explode, catch fire, smoke or leak.

电池不应爆炸、起火、冒烟或泄漏

6.5.3. External Short-circuit Test外部短路测试
•  Test Method测试方法:

After charging at 0.2C5A at an ambient temperature of 25±5°C; short-circuit the

positive and negative poles with a 0.1Ω resistor for 1.5 hours; after the short circuit

is disconnected, charge for 10 seconds with a current of 0.2C5A, and then

Measure the battery voltage.

在25±5°C环境温度下以0.2C5A充满电后，用0.1Ω的电阻将正负极短路1.5小时。短路断开后，以0.2C5A的电流充电10秒，然后测量电池电压。

•  Approval Standard合格标准:

Battery voltage discharge cut-off voltage; the battery should not explode, fire,

smoke or leak.

电池电压≧放电截止电压;电池应不爆炸，不起火，不冒烟或漏液。

7. Transportation, Storage运输、存储

7.1. Transportation运输

The battery should be packed into a box for transportation at a 30% load.

During  the transportation, it should prevent severe vibration, impact or

squeeze, prevent sun and rain, and adapt to the transportation of sea, land

and air.

电池应按30%的带电量装入箱内运输。在运输过程中，应防止剧烈振动、撞击或挤压，防止日晒雨淋，适应海、陆、空的运输。

Tel : +86 158 1732 3917

Email :cherry@cmbatteries.com

CMB Headquarters：
Rm.1216 ,

Baoshan Times Bldg,

Minqiang Community,

Minzhi St.,Longhua Dist.,
Shenzhen

GuangdongChina

CMB Facotry：

Rm.702,7/F,4 BLDG,NO.160,

Tianxin Luyuan Rd.,

Keyuan City,Tangxia Town,

Dongguan,

Guangdong,China

Date 日期：

20250716

Project Name项目名：

CMB04040173

Manufacturer

生产制造商

CM Batteries Co.,Ltd

Customer ID

客户编码：

ABXXX

page：13

PRODUCT SPECFICATION   产品规格书

7.2. Storage存储

The battery should be stored in a clean and dry and ventilated room with a ambient temperature of -20° C to +20 ° C. It should avoid contact with corrosive

substances and stay away from the fire source and thermal source.

电池应存放在清洁、干燥、通风的室内，环境温度为-20℃至+20℃。应避免接触腐蚀性物质，并远离火源和热源。

- 8.Handling Instructions操作说明

Read the following precautions carefully to ensure the correct use of lithium ion

batteries.CM BATTERIES is not responsible for any problems arising from the

following precautions.

仔细阅读以下预防措施，以确保正确使用锂离子电池。CM BATTERIES不负责任何引起的任何问题负责。

Danger! 危险！

•  Do not put the battery into the water or wet it!

请勿将电池放入水中或将其弄湿!

•  It is forbidden to charge the battery under the source of fire or extremely heat! Do not use or store batteries near the heat source (such as fire or heater)! If thebattery leaks or produces a odor, it should be immediately removed from the
open flame;

严禁在火源或极热的情况下给电池充电! 不要在热源（如火或加热器）附近使用或储存电池! 如果电池发生泄漏或产生异味，应立即将其从明火上移开。

•  Do not use any chargers other than those recommended!

请不要使用推荐以外的任何充电器!

•  Do not reverse the positive (+) and negative (-) terminals.

不要把电池的正(+)和负(-)接反。

•  Do not put the battery into the fire or heat it to the battery!
不要把电池放入火中或加热电池!

•  Do not short-circuit the battery by connecting wires or other metal objects to

the positive (+) and negative (-) terminals.

不要用电线或其他金属物体短接电池的正极（+）和负极（-），使电池短路。

•  Do not pierce the battery casing with a nail or other sharp object, break

it open with a hammer, or step on it.

不要用钉子或其他尖锐物体刺穿电池外壳，不要用锤子砸开电池外壳，也不要踩在上面。

•  Do not strike,throw or subject the battery to sever physical shock.

请勿敲击、投掷或使电池受到严重的物理冲击。

•  Do not directly solder the battery terminals.

不要直接焊接电池端子。

•  Do not attempt to disassemble or modify the battery in any way.

不要试图以任何方式拆卸或修改电池。

•  Do not place the battery in a microwave oven or pressurized container.

不要把电池放在微波炉或加压的容器中。

Tel : +86 158 1732 3917

Email :cherry@cmbatteries.com

CMB Headquarters：
Rm.1216 ,

Baoshan Times Bldg,

Minqiang Community,

Minzhi St.,Longhua Dist.,
Shenzhen

GuangdongChina

CMB Facotry：

Rm.702,7/F,4 BLDG,NO.160,

Tianxin Luyuan Rd.,

Keyuan City,Tangxia Town,

Dongguan,

Guangdong,China

Date 日期：

20250716

Project Name项目名：

CMB04040173

Manufacturer

生产制造商

CM Batteries Co.,Ltd

Customer ID

客户编码：

ABXXX

page：14

PRODUCT SPECFICATION   产品规格书

• Do not use the battery in combination with primary batteries (such as dry-cell
batteries) or batteries of different  Capacity type or brand.

请勿将电池与原电池（如干电池）或不同容量类型或品牌的电池一起使用。

• Do not use the battery if it gives off an odor, generates heat, becomes discolored

or deformed, or appears abnormal in any way. If the battery is in use or being

recharged, remove it from the device or charger immediately and discontinue use.

如果电池发出异味，产生热量，变色或变形，或出现任何异常，请不要使用。如果电池正在使用或正在充电，请立即将其从设备或充电器中取出，并停止使用。

Caution！注意事项！

Do not use or store the battery where is exposed to extremely hot, such as under window of a car indirect sunlight in a hot day. Otherwise, the battery may be overheated. This can also reduce battery performance and/or shorten cycle life.

If the electrolyte enters the eyes after the battery leaks, do not wipe it, apply
water to rinse, and immediately seek medical assistance. If you do not deal with

it in time, your eyes will be harmed.

The battery can only be used under the following conditions, otherwise it will
reduce the performance of the battery or shorten the service life of the battery.

不要在极热的地方使用或存放电池。例如高温天气下，搁置在阳光直射的汽车窗下。否则，电池可能会过热。这也会降低电池性能和/或缩短循环寿命。

如果电池泄漏后，电解液进入眼睛，不要擦拭，用清水冲洗，并立即寻求医疗援助。如果不及时处理，眼睛会受到伤害。

电池只能在以下条件下使用，否则会降低电池的性能或缩短电池的使用寿命。

Operating Environment工作环境:

Charge充电：0 ° C~+45 ° C
Discharge放电：-20° C~+60° C
Storage for 30 days储存30天：-20 ° C~+35 ° C
Storage for 90 days 储存大于90天：-20 ° C~+20 ° C

- 9. Amendment of this Specification产品规格书的修订

This specification is subject to change with prior notice.

本公司有权对本产品规格书进行修订。

Tel : +86 158 1732 3917

Email :cherry@cmbatteries.com

CMB Headquarters：
Rm.1216 ,

Baoshan Times Bldg,

Minqiang Community,

Minzhi St.,Longhua Dist.,
Shenzhen

GuangdongChina

CMB Facotry：

Rm.702,7/F,4 BLDG,NO.160,

Tianxin Luyuan Rd.,

Keyuan City,Tangxia Town,

Dongguan,

Guangdong,China

Date 日期：

20250716

Project Name项目名：

CMB04040173

Manufacturer

生产制造商

CM Batteries Co.,Ltd

Customer ID

客户编码：

ABXXX

page：15

PRODUCT SPECFICATION   产品规格书